- REPEATEDLY PARTIALLY PRESSING AND RELEASING THE BRAKES PEDAL MAY RESULT IN:

- A LOSS OF BRAKE AIR PRESSURE.

- A BUILD UP OF BRAKE AIR PRESSURE.

- NO CHANGE OF BRAKE AIR PRESSURE.

- IF YOU MUST MAKE AN EMERGENCY STOP, BRAKE SO YOU:

- CAN STEER HARD WHILE BRAKING HARD.

- CAN STEER AND SO YOUR VEHICLE STAYS IN A STRAIGHT LINE.

- USE THE HAND BRAKE FIRST.

- THE MOST COMMON TYPE OF FOUNDATION BRAKE ON HEAVY VEHICLES IS THE:

- WEDGE DRUM.

- S-CAM DRUM.

- DISC BRAKE.

- YOU HAVE A MAJOR LEAK IN THE SERVICE LINE AND PUT ON THE BRAKES. SERVICE AIR PRESSURE WILL ESCAPE AND CAUSE THE:

- TRAILER TANK PRESSURE TO BE LOST.

- TRACTOR SPRING BRAKES TO LOCK ON.

- TRAILER EMERGENCY BRAKES TO COME ON.

- IF YOUR TRUCK HAS A PROPERLY FUNCTIONING DUAL AIR BRAKE SYSTEM AND MINIMUM SIZE AIR TANKS THE AIR PRESSURE SHOULD BUILD UP FROM 85 TO 100 PSI WITHIN HOW MANY SECONDS?

- 30.

- 45.

- 60.

- THE PROPER USE OF THE BRAKES WHEN GOING DOWN A LONG STEEP GRADE AFTER SELECTING A PROPER GEAR IS TO BRAKE UNTIL YOUR SPEED IS ABOUT _______ MPH BELOW THE POSTED SPEED FOR YOUR SAFETY AND RELEASE YOUR BRKAES.

- 5.

- 10.

- 15.

- TO CHECK THE FREE PLAY OF MANUAL SLACK ADJUSTERS OF S-CAM BRAKES, YOU SHOULD PARK ON:

- LEVEL GROUND, CHOCK THE WHEELS AND RELEASE THE PARKING BRAKES.

- LEVEL GROUND AN APPLY THE PARKING BRAKES, THEN APPLY SERVICE BRAKES.

- LEVEL GROUND AND DRAIN OFF AIR PRESSURE BEFORE CHECKING THE ADJUSTMENT.

- IF YOUR VEHICLE HAS AN ALCOHOL EVAPORATOR, IT IS HERE TO:

- ELIMINATE THE NEED FOR DAILY TANK DRAINING.

- BOOST TANK PRESSURE THE SAME WAY THAT TURBOCHARGES BOSST ENGINE.

- REDUCE THE RISK OF ICING IN AIR BRAKE VALVES IN COLD WEATHER.

- YOUR TRUCK HAS A DUAL AIR SYSTEM AND ONE OF THE SYSTEM LOSES ITS PRESSURE. WHAT WILL HAPPEN?

- BRAKE DRUMS WILL NOT BE FULLY PRESSURIZED.

- THE MANUAL SLACK ADJUSTERS OF THE S-CAM BRAKES WILL NOT BE SET PROPERLY.

- EITHER THE FRONT OR BACK BRAKE WILL NOT BE FULLY OPERATIONAL.

- WHY SHOULD YOU DRAIN WATER FROM COMPRESSED AIR TANKS?

- WATER LOW BOILING POINT REDUCES BRAKING POWER.

- WATER CAN FREEZE IN COLD WEATHER AND CAUSE BRAKE FAILURE.

- TO KEEP HEAT FROM AIR COMPRESSOR OIL.

- IF YOUR TRUCK OR BUS HAS DUAL PARKING CONTROL VALVES, YOU CAN USE PRESSURE FROM A SEPARATE TANK TO:

- BALANCE THE SERVICE BRAKE SYSTEM WHEN YOU ARE PARKED.

- STAY PARKED TWICE AS LONG WITHOUT USING UP SERVICE AIR PRESSURE.

- RELEASE THE SPRING EMERGENCY/PARKING BRAKES TO MOVE A SHORT DISTANCE.

- IF YOU MUST MAKE AN EMERGENCY STOP, YOU SHOULD BRAKE SO YOU:

- USE THE HAND BRAKE BEFORE THE BRAKE PEDAL.

- CAN STEER AND SO YOUR VEHICLE STAYS IN A STRAIGHT LINE.

- USE THE FULL POWER OF THE BRAKES TO LOCK THEM.

- THE SUPPLY PRESSURE GAUGE SHOWS HOW MUCH PRESSURE:

- IS IN THE AIR TANKS.

- YOU HAVE USED IN THIS TRIP.

- IS GOING TO THE BRAKE CHAMBERS.

- THE APPLICATION PRESSURE GAUGE SHOWS HOW MUCH AIR PRESSURE YOU:

- HAVE IN THE AIR TANKS.

- ARE APPLYING TO THE BRAKES.

- HAVE IN A MODULATING CONTROL VALVE.

- THE SPRING BRAKES USED ON TRACTORS AND STRAIGHT TRUCKS WILL BRING YOUR TRUCK TO A STOP WHEN THE AIR PRESSURE DROPS BELOW __________PSI.

- 20.

- 60.

- 100.

- THE AIR LEAKAGE RATE FOR COMBINATION VEHICLES (ENGINE OFF, BRAKES ON) SHOULD BE LESS THEN _____ PSI PER MINUTE.

- 2.

- 3.

- 4.

- HOW SHOULD YOU CHECK THAT YOUR SERVICE BRAKES ARE WORKING PROPERLY?

- PARK ON SLIGHT GRADE, DRAIN OFF AIR PRESSURE, SET PARKING BRAKE AND CHECK TO THE MOVEMENT.

- PARK ON A LEVEL GROUND, CHOCK THE WHEELS, ENGAGE THE PARKING BRAKES WHEN YOU HAVE THE CORRECT AMOUNT OF AIR PRESSURE TO DO SO AND SHUT THE ENGINE OFF.

- PARK ON LEVEL GROUND, WAIT UNTIL NORMAL AIR PRESSURE IS REACHED, RELEASE THE PARKING BRAKE AND MOVE FORWARD SLOWLY AT ABOUT 5 MPH. THEN APPLY THE BRAKES FIRMLY USING THE BRAKE PEDAL.

- EXCESSIVE USE OF THE SERVICE BRAKES RESULTS IN OVEHEATING WHICH CAN LEAD TO:

- IMPROPER ADJUSTMENT OF THE S-CAM.

- INCREASED CONTACT BETWEEN THE BRAKE DRUMS AND BRAKE LINNNING.

- EXPANSION OF THE BRAKE DRUMS.

- THE SUPPLY PRESSURE GAUGE SHOWS HOW MUCH PRESSURE:

- IS IN THE AIR TANKS.

- TRACTOR BRAKE LINES.

- TRAILER BRAKE LINES.

- TOTAL STOPPING DISTANCE FOR AIR BRAKES IS LONGER THAN THAT FOR HYDRAULIC BRAKES DUE TO _________ DISTANCE.

- BRAKE LAG.

- REACTION.

- EFFECTIVE BRAKING.

- THE USE OF AIR BRAKES ON LONG AND STEEP DOWNGRADE UNDER NORMAL CONDITIONS IS ONLY A SUPPLEMENT TO:

- THE USE OF THE SPRING BRAKES.

- THE USE OF THE FRONT BRAKES LIMITING VALVE.

- THE BRAKING EFFECT OF THE ENGINE.

- ANY TIME YOU PARK, USE THE PARKING BRAKES, EXCEPT:

- IF THE BRAKES ARE VERY HOT (E.G., AFTER JUST COMING DOWN A STEEP GRADE).

- IF YOU ARE GOING TO TEST THE PARKING BRAKE MAKE SURE THAT IT HOLDS THE VEHICLE.

- IF YOU WILL ONLY BE STOPPED FOR LESS THEN ONE HOUR.

- IF YOUR VEHICLE GETS STUCK ON A RAISED RAILROAD- HIGHWAY CROSSING, YOU SHOULD:

- GET OUT OF THE VEHICLE AND AWAY FROM THE TRACKS.

- TURN ON YOUR FOUR- WAYS FLASHERS.

- USE REVERSE GEAR TO BACK UP OFF THE CROSSING.

- YOUR TRUCK OR BUS HAS A DUAL AIR BRAKE SYSTEM. IF A LOW AIR PRESSURE WARNING COMES ON FOR ONLY ONE SYSTEM, WHAT SHOULD YOU DO?

- REDUCE YOUR SPEED, AND DRIVE TO THE NEAREST GARAGE FOR REPAIRS.

- REDUCE YOUR SPEED, AND TEST THE REMANING SYSTEM WHILE UNDER WAY.

- STOP RIGHT AWAY AND SAFELY PARK. CONTINUE ONLY AFTER THE SYSTEM IS FIXED.

- YOUR TRUCK OR BUS HAS A DUAL AIR BRAKE SYSTEM. IF A LOW AIR PRESSURE WARNING COMES ON FOR ONLY ONE SYSTEM, WHAT SHOULD YOU DO?

- BRING YOUR VEHICLE TO COMPLETE STOP RIGHT AWAY AND SAFELY PARK. CONTINUE ONLY AFTER THE SYSTEM IS FIXED.

- REDUCE YOU SPEED, AND TEST THE REMAINING SYSTEM WHILE UNDER WAY.

- REDUCE YOUR SPEED, AND DRIVE TO THE NEAREST GARAGE FOR REPAIRS.

- THE SAFETY VALVE IS SET TO AUTOMATICALLY REDUCE PRESSURE AT _____PSI.

- 50.

- 100.

- 150.

- SOME AIR BRAKE SYSTEM HAVE AN ALCOHOL EVAPORATOR. WHAT MAY HAPPEN IF YOU DON’T KEEP THE PROPER LEVEL OF AN ALCOHOL?

- THE S-CAM MAY NOT TAKE BACK WHEN YOU RELEASE THE BRAKE PEDAL.

- ICE MAY FORM IN THE AIR STORAGE TANK AND CAUSE A BRAKE FAILURE.

- ICE MAY FORM ON THE BRAKE DRUMS AND WEAR THEM OUT.

- UNDER NORMAL CONDITIONS IN ORDER TO ENGAGE THE PARKING BRAKES DRIVER:

- TURN OFF THE ENGINE.

- LET THE AIR OUT OF AIR BRAKE SYSTEM.

- BE SURE THAT THE AIR BRAKE SYSTEM IS FULLY PRESSURIZED.

- PARKING OR EMERGENCY BRAKES OF TRUCKS AND BUSES CAN BE LEGALLY HELD ON BY ____ PRESSURE.

- SPRING.

- AIR.

- FLUID.

- ALL AIR BRAKE EQUIPPED VEHICLES HAVE:

- A SUPPLY PRESSURE GAUGE.

- AN AIR USAGE GAUGE.

- A BACKUP HYDRAULIC SYSTEM.

- THE MODULATING CONTROL VALVE ALLOWS YOU TO CONTROL:

- SPRING BRAKES.

- FRONT BRAKES.

- AMOUNT OF AIR PRESSURE IN THE BRAKE SYSTEM.

- A SLACK ADJUSTERS FREE PLAY NEEDS TO BE ADJUSTED IF IT IS MORE THAN ABOUT______ HOW MANY INCHES WHEN YOU PULL HARD ON IT?

- ONE FOURTH INCH.

- ONE HALF INCH.

- ONE INCH.

- EMERGENCY STAB BRAKING IS:

- PRESS HARD ON A BRAKE PEDAL AND APPLY HAND VALVE UNTIL YOU STOP.

- USE LIGHT STEADY PRESSURE ON THE BRAKE PEDAL.

- BRAKE AS HARD AS YOU CANY, RELEASE THE BRAKES WHEN THE WHEELS LOCKED, PUT ON THE BRAKES AGAIN WHEN THE WHEELS START ROLLING.

- IN IDEAL CONDITIONS, A TRUCK OR A BUS WITH AN AIR BRAKE GOING AT 55 MPH WOULD REQUIRE STOPPING DISTANCE OF:

- LESS THAN 100 FEET.

- 100-300 FEET.

- MORE THAN 300 FEET.

- TO TEST AIR SERVICE BRAKES, YOU SHOULD:

- STOP THE VEHICLE, PUT IT IN A LOW GEAR, DEPRESS THE SERVICE BRAKES, AND THEN GENTLY PULL AGAINST THE BRAKES.

- BRAKE FIRMLY WHILE SLOWLY MOVING FORWARD.

- BRAKE SLOWLY WHILE SLOWLY MOVING BACKIWARD.

- THE S-CAM:

- CONTROLS THE FLOW AIR TO EACH OF THE BRAKE CHAMBERS.

- PULLS THE BRAKE SHOES AWAY FROM THE DRUM AND ALLOW THE WHEELS TO ROLL FREELY.

- FORCES THE BRAKE SHOES AGAINST THE INSIDE OF THE BRAKE DRUM.

- WHY SHOULD YOU DRAIN WATER FROM COMPRESSED AIR TANKS?

- THE BOILING POINT REDUCES BRAKING POWER.

- WATER CAN FREEZE IN COLD WEATHER AND CAUSE BRAKE FAILURE.

- WATER COOLS THE COMPRESSOR TOO MUCH.

- THE AIR BRAKE LAG DISTANCE AT 55 MPH ON DRY PAVEMENT ADDS ABOUT ______ FEET.

- 12 FEET.

- 32 FEET.

- 52 FEET.

- WHEN IS IT OK TO LEAVE YOUR TRUCK UNATTENDED WITHOUT APPLYING THE PARKING BRAKES AND CHOCKING THE WHEELS?

- NEVER.

- IF YOU WILL BE AWAY FROM THE TRUCK FOR A FEW MINUTES.

- IF YOU ARE CONDUCTING A PRE-TRIP INSPECTION.

- THE BRAKE SYSTEM THAT APPLIES AND RELEASES THE BRAKES WHEN THE DRIVER USES THE BRAKE PEDAL IS THE ________ BRAKE SYSTEM.

- EMERGENCY.

- SERVICE.

- PARKING.

- WHICH OF THESE IS NOT A PROPER TIME TO APPLY THE PARKING BRAKES?

- TO BRAKE VEHICLE VERY HARD, IN OTHER WORD WHEN COMING DOWN THE STEEP GRADE.

- TO USE PARKING BRAKES IF YOU PARK FOR LESS THAN 1 HOUR.

- IF YOU ARE GOING TO USE THE PARKING BRAKES YOU NEED TO MAKE SURE THAT THEY WILL HOLD THE VEHICLE.

- AIR BRAKING TAKES MORE TIME THAN HYDRAULIC BRAKING BECAUSE AIR:

- BRAKE USE DIFFERENT BRAKE DRUMS.

- TAKES MORE TIME TO FLOW THROUGH THE LINES THE HYDRAULIC FLUID.

- BRAKES REQUIRE HEAVIER RETURN SPRINGS.

- A STRAIGHT TRUCK OR BUS BRAKE SYSTEM SHOULD NOT LEAK AT THE RATE OF MORE THAN _____ PSI PER MINUTE WITH THE ENGINE OFF AND THE BRAKES RELEASED.

- ONE.

- THREE.

- TWO.

- WHY SHOULD YOU DRAIN WATER FROM COMPRESSED AIR TANKS?

- THE LOW BOILING POINT OF WATER REDUCES BRAKING POWER.

- WATER CAN FREEZE IN COLD WEATHER AND CAUSE BRAKE FAILURE.

- TO KEEP FROM FOWLING TRE AIR COMPRESSOR OIL.

- IF THE SPRING BRAKE ARE ON, WHEN SHOULD YOU PUSH THE BRAKE PEDAL?

- ONLY WHEN DRIVING DOWN A HILL.

- ONLY ON THE SLIPPERY ROAD

- NEVER.

- WITH AIR BRAKE VEHICLE, THE PARKING BRAKES SHOULD BE USED:

- WHENEVER YOU LEAVE THE VEHICLE UNATTENDED.

- AS LITTLE AS POSSIBLE.

- ONLY DURING PRE- AND POST TRIP INSPECTIONS.

- THE MOST IMPORTANT THING TO DO WHEN A LOW AIR PRESSURE WARNING COMES ON IS:

- UPSHIFT.

- DOWNSHIFT.

- STOP AND SAFELY PARK AS SOON AS POSSIBLE.

- DURING NORMAL DRIVING, SPRING BRAKES ARE USUALLY HELD BACK BY:

- BOLTS.

- AIR PRESSURE.

- SPRING PRESSURE.

- IF YOU ARE DRIVING DOWN A STEEP DOWNGRADE AND YOU HAVE REACHED YOUR ‘’SAFE’’ SPEED OF 40 MPH YOU WOULD APPLY THE SERVICE BRAKES UNTIL YOUR SPEED DROPS TO ___ MPH.

- 25.

- 30.

- 35.

- THERE ARE TWO THINGS THAT A DRIVER CAN DO TO PREVENT A ROLLOVER. THEY ARE: (1) KEEP THE CARGO AS CLOSE TO THE GROUND AS POSSIBLE; AND (2):

- GO SLOW AROUND TURNS.

- KEEP BOTH HANDS FIRMLY ON THE STEERING WHEEL.

- MAKE SURE THAT THE BRAKES ARE PROPERLY ADJUSTED.

- EMERGENCY CONTROLLED BRAKING IS WHEN YOU:

- APPLY THE BRAKES AS HARD AS YOU CAN WITHOUT LOCKING THE WHEELS.

- BRAKE AS HARD AS YOU CAN, RELEASE THE BRAKES WHEN WHEELS LOCKING, BRAKES BACK ON AGAIN WHEN THE WHEELS START ROLLING.

- APPLY THE HAND VALVE FOR ONE SECOND, THEN PUSH HARD ON THE BRAKES.

- WHEN BRAKES ARE APPLIED THE BRAKE SHOES (OR LININGS) WILL ALIGN TO PRESS AGAINST THE:

- BRAKE DRUM OR DISC.

- SLACK ADJUSTER.

- S-CAM.

- THE STOP LIGHT SWITCH:

- TELLS YOU WHEN THE AIR BRAKE SYSTEM IS LOW ON AIR PRESSURE.

- TELLS YOU WHEN YOU NEED TO USE YOUR EMERGENCY BRAKES.

- TURNS ON YOUR BRAKE LIGHTS TO WARN DRIVERS BEHIND YOU.

- IF YOUR VEHICLE HAS AN ALCOHOL EVAPORATOR, EVERY DAY DURING COLD WEATHER YOU SHOULD:

- CHECK AND FILL THE ALCOHOL LEVEL.

- CHANGE THE ALCOHOL FROM A NEW BOTTLE.

- CHECK THE OIL FOR ALCOHOL CONTENT.

- THE BRAKE PEDAL IN AN AIR BRAKE SYSTEM:

- CONTOLS THE SPEED OF THE AIR COMPRESSOR.

- CONTROLS THE AIR PRESSURE APPLIED TO PUT ON THE BRAKES.

- IS CONNECTED TO SLACK ADJUSTERS BY A SERIES OF RODS AND LINKAGES.

- IT IS NOT SAFE TO DRIVE A VEHICLE THAT HAS BRAKE DRUMS WITH CRACK THAT ARE LONGER THAN ______ OF THE WIDTH OF THE FRICTION AREA.

- ONE HALF.

- ONE FOURTH.

- ONE EIGHT.

- BEFORE DRIVING A TRUCK OR BUS WITH A DUAL AIR BRAKE SYSTEM, THE AIR PRESSURE MUS BE AT LEAST____ PSI.

- 80.

- 100.

- 120.

- AN AIR BRAKE SYSTEM IS FULLY CHARGED AT____ PSI.

- 75.

- 100.

- 125.

- A STRAIGHT TRUCK OR BUS AIR BRAKE SYSTEM SHOULD NOT LEAK AT THE RATE OF MORE THAN ______ PSI PER MINUTE WITH THE ENGINE OFF AND THE BRAKES RELEASED.

- 1.

- 2.

- 3.

- THE AIR COMPRESSOR GOVERNOR CONTROLS:

- THE SPEED OF THE AIR COMPRESSOR.

- AIR PRESSURE APPLIED TO THE BRAKES.

- WHEN AIR IS PUMPED INTO THE AIR TANKS.

- WHY DRAIN WATER FORM COMPRESSED AIR TANKS?

- THE LOW BOILING POINT OF WATER REDUCES BRAKING POWER.

- WATER CAN FREEZE IN COLD WEATHER AND CAUSE BRAKE FAILURE.

- WATER COOLS THE COMPRESSOR TOO MUCH.

- EXCESSIVE HEAT CAUSED BY USING YOUR BRAKE TOO OFTEN CAN ALSO CAUSE:

- MODULATING CONTROL VALVE TO WEAR OUT.

- BRAKE LINES TO PLUG.

- BRAKES TO FADE OR FAIL.

- THE AIR COMPRESSOR SHOULD STOP PUMPING AIR WHEN PRESSURE REACH____PSI.

- 100.

- 125.

- 150.

- THE AIR LOSS RATE FOR A STRAIGHT TRUCK OR BUS WITH THE ENGINE OFF AND THE BRAKES ON SHOULD NOT BE MORE THAN:

- 1 PSI IN ONE MINUTE.

- 2 PSI IN 45 SECONDS.

- 3 PSI IN ONE MINUTE.

- THE SAFETY VALVE DISCHARGE AUTOMATICALLY AT THE PRESSURE OF:

- 50.

- 100.

- 150.

- YOUR BRAKES ARE FADING WHEN:

- YOU HAVE TO PUSH HARDER ON THE BRAKE PEDAL TO CONTOL YOUR SPEED ON A DOWNGRADE.

- LESS PRESSURE IS NEEDED ON THE BRAKE PEDARL FOR EACH STOP.

- THE BRAKE PEDAL FEEL SPONGY WHEN YOU APPLY PRESSURE.

- IN AIR BRAKES VEHICLES, THE PARKING BRAKES SHOULD BE USED:

- AS LITTLE AS POSSIBLE.

- ANY TIME WHEN VEHICLE IS PARKED.

- TO HOLD YOUR SPEED WHEN GOING DOWNHILL.

- THE BRAKING POWER OG THE SPRING BRAKES:

- IS NOT AFFECTED BY THE CONDITION OF THE SERVICE BRAKES.

- DEPENDS ON THE SERVICE BRAKES BEIGN IN ADJUSTMENT.

- INCREASES WHEN THE SERVICE BRAKES ARE HOT.

- IF AIR PRESSURE IS NOT BUILT UP WITHIN THE CORRECT AMOUNT OF TIME, THEN:

- YOU SHOULD BE READY TO USE YOUR PARKING BRAKES TO HELP STOP.

- THE ALCOHOL CONTAINER MAY BE LOW.

- YOUR AIR PRESSURE MAY DROP TOO A LOW DURING DRIVING, REQUIRING AN EMERGENCY STOP.

- THE SAFETY VALVE REDUCES PRESSURE AT ____ PSI?

- 50.

- 100.

- 150.

- WHEN BRAKES ARE APPLIED THE BRAKE SHOES WILL PRESS AGAINST THE:

- BRAKE DRUM OR DISC.

- SLACK ADJUSTER.

- S-CAM.

- THE DRIVER MUST BE ABLE TO SEE A LOW AIR PRESSURE WARNING WHICH COMES ON BEFORE PRESSURE IN THE SERVICE AIR TANKS FALLS BELOW _____ PSI.

- 40.

- 60.

- 80.

- THE PARKING OR EMERGENCY BRAKE ON A HEAVY VEHICLE CAN ONLY BE HELD IN POSITION BY SOMETHING THAT CANNOT LEAK AWAY, LIKE:

- SPRING PRESSURE.

- AIR PRESSURE.

- HYDRAULIC PRESSURE.

- IT IS ACCEPTED THAT TOO MUCH HEAT CAUSED BY USING YOUR BRAKES TOO OFTEN CAN ALSO CAUSE:

- MODULATED CONTROL VALVE TO WEAR OUT.

- BRAKE LININGS TO SLIPT UP.

- BRAKE TO FADE OR FAIL.

- A SLACK ADJUSTER PROBABLY NEEDS TO BE ADJUSTED IF IT MOVES MORE THAN_______ INCHES WHEN YOU PULL HARD ON IT?

- ¼.

- ½.

- 1.

- MODERN AIR BRAKE SYSTEM COMBINE THREE DIFFERENT SYSTEMS. THE ARE THE SERVICE, THE PARKING AND THE ______ BRAKES.

- EMERGENCY.

- FOOT.

- S-CAM.

- IF YOU DO NOT HAVE AUTOMATIC TANK DRAINS, HOW OFTEN SHOULD YOU DRAIN THE OIL AND WATER FROM THE BOTTOM OF COMPRESSED AIR STORAGE TANKS?

- AFTER EVERY FOUR HOURS OF SERVICE.

- AT THE END OF EACH DAY OF DRIVING.

- ONCE A WEEK.

- YOUR VEHICLE HAS A DUAL AIR BRAKE SYSTEM. IF A LOW AIR PRESSURE WARNING COMES ON FOR ONLY ONE SYSTEM, YOU SHOULD:

- BRING THE VEHICLE TO A SAFE STOP AND CONTINUE ONLY AFTER THE SYSTEM IS FIXED.

- REDUCE YOUR SPEED, AND TEST THE REMAINING SYSTEM WHILE UNDER WAY.

- REDUCE YOUR SPEED, AND DRIVE TO THE NEAREST GARAGE FOR REPAIRS.