- WHILE CHECKING IF THE TRAILER IS SECURELY COUPLED TO THE TRAILER THE LANDING GEARS SHOULD BE:

- FULLY RAISED.

- SLIGHTLY RAISED.

- FULLY LOWERED.

- IN GENERAL, THE HIGHER THE ‘’CENTER OF GRAVITY’’ OF YOUR TRUCK, THE:

- EASIER IT IS TO TURN AROUND CORNERS.

- MORE STABLE IT IS WHEN TURNING.

- EASIER IT IS TO TURN OVER.

- WHY SHOULD YOU BE SURE THAT THE FIFTH WHEEL PLATE IS GREASED AS REQUIRED?

- TO ENSURE A GOOD ELECTRICAL CONNECTION.

- TO PREVENT STEERING PROBLEMS.

- TO REDUCE HEAT AND NOISE.

- BEFORE YOU BACK UNDER A TRAILER, MAKE SURE THAT:

- THE TRAILER BRAKES ARE LOCKED.

- THE AIR BRAKES ARE OFF.

- THE AIR SUPPLY KNOB IS IN.

- IF THE SPRING BRAKES DON’T RELEASE WHEN YOU PUSH THE TRAILER AIR SUPPLY VALVE, YOU SHOULD:

- CHECK AIR LINE CONNECTIONS.

- CROSS THE AIR LINES.

- CHECK THE ELECTRICAL SERVICE CABLE.

- AFTER CONNECTING THE AIR LINES BUT BEFORE BACKING UNDER THE TRAILER, YOU SHOULD:

- SUPPLY AIR TO THE TRAILER SYSTEM, THEN PULL OUT THE AIR SUPPLY KNOB.

- MAKE SURE THAT THE TRAILER BRAKES ARE OFF.

- WALK ARROUND THE RIG TO BE SURE THAT IT IS CLEAR.

- IF THE SERVICE AIR LINE COMES APART WHILE YOU ARE DRIVING A COMBINATION VEHICLE BUT THE EMERGENCY LINE STAYS TOGETHER, WHAT WILL HAPPEN RIGHT AWAY?

- THE TRAILER’S TANK WILL EXHAUST THROUGH THE OPEN LINE.

- THE EMERGENCY TRAILER BRAKES WILL COME ON.

- NOTHING IS LIKELY TO HAPPEN UNTIL YOU TRY TO APPLY THE BRAKES.

- THE FRONT TRAILER SUPPORTS ARE UP AND THE TRAILER IS RESTING ON THE TRACTOR. MAKE SURE:

- THERE IS ENOUGH CLEARANCE BETWEEN THE UPPER AND LOWER FIFTH WHEEL.

- THE SAFETY LATCH IS IN THE UNLOCKED POSITION.

- THERE IS ENOUGH CLEARANCE BETWEEN THE TRACTOR FRAME AND THE LANDING GEAR.

- YOUR EMERGENCY AIR LINE BREAKS OR GETS PULLED APART WHILE YOU ARE DRIVING. THE LOSS OF PRESSURE WILL CAUSE THE:

- AIR COMPRESSOR TO UNLOAD INSTEAD OF PUMPING AIR.

- TRACTOR’S AIR TO DUMP INTO THE TRAILER TANK THROUGH THE SERVICE LINE.

- EMERGENCY TRAILER BRAKES TO COME ON.

- YOU HAVE A MAJOR LEAK IN THE SERVICE LINE AND YOU PUT ON THE BRAKES. SERVICE AIR WILL ESCAPE AND CAUSE THE:

- TRAILER EMERGENCY BRAKES TO COME ON.

- TRAILER TANK PRESSURE TO BE LOST.

- TRACTOR SPRING BRAKES TO LOCK ON.

- THE TRAILER AIR SUPPLY CONTROL VALVE SHOULD AUTOMATICALLY CLOSE WHEN AIR PRESSURE FALLS TO BETWEEN _______PSI.

- 20 AND 45.

- 60 AND 85.

- 100 AND 125.

- YOU HAVE CRASHED WITH ANOTHER VEHICLE. WHAT YOU HAVE TO DO FIRST?

- USE REVERSE GEAR TO BACK UP OFF THE CRASH.

- TURN ON YOUR FOUR-WAY FLASHERS.

- GET OUT OF YOUR VEHICLE AND AWAY FROM THE SCENE.

- YOU HAVE PUSHED IN THE TRAILER AIR SUPPLY VALVE. YOU SHOULD NOT BACK YOUR TRACTOR UNDER THE TRAILER UNTIL WHOLE AIR SYSTEM IS:

- EMPTY.

- AT NORMAL PRESSURE.

- BETWEEN 60 AND 80 PSI.

- THE FIFTH WHEEL LOCKING LEVER IS NOT LOCKED AFTER THE JAWS CLOSE AROUND THE KINGPIN. THIS MEANS THAT:

- THE TRAILER WILL NOT SWIVEL ON THE FIFTH WHEEL.

- THE PARKING LOCK IS OFF AND YOU MAY DRIVE AWAY.

- THE COUPLING IS NOT RIGHT AND SHOULD BE FIXED BEFORE DRIVING THE COUPLED UNIT.

- WHEN BACKING A TRACTOR UNDER A TRAILER YOU SHOULD:

- ALWAYS USE THE LOWEST REVERSE GEAR.

- ALWAYS APPROACH THE TRAILER AT A SLIGHT ANGLE.

- DO IT QUICKLY TO ENSURE THAT THE KINGPIN IS LOCKED INTO THE FIFTH WHEEL.

- IN NORMAL DRIVING, SOME DRIVERS USE THE HAND VALVE BEFORE THE BRAKE PEDAL TO PREVENT A JACKKNIFE. WHICH OF THESE STATEMENTS IS TRUE?

- IT SHOULD NOT BE DONE.

- IT RESULTS IN LESS SKIDDING THAT USING THE BRAKE PEDAL ALONE.

- IT IS THE BEST WAY TO BRAKE AND KEEP THE VEHICLE IN A STRAIGHT LINE.

- WHEN YOU ARE UNCOUPLING A LOADED TRAILER, YOU SHOULD LOWER THE LANDING GEAR UNTIL IT:

- MAKES FIRM CONTACT WITH THE GROUND, THEN CRANK IT IN LOW GEAR A FEW EXTRA TURNS.

- MAKE CONTACT WITH THE GROUND AND LIFTS THE TRAILER OFF THE FIFTH WHEEL.

- JUST REACHES THE GROUND.

- WHEN YOU GET READY TO BACK UNDER A SEMITRAILER YOU SHOULD LINE UP:

- THE KINGPIN TO ENGAGE THE DRIVER’S SIDE LOCKING JAW FIRST.

- DIRECTLY IN FRONT OF THE TRAILER.

- THE LEFT REAR OUTER DUAL WHEEL WITH THE KINGPIN.

- YOU HAVE COUPLED WITH A SEMITRAILER. WHEN SHOULD YOU PUT THE FRONT TRAILER SUPPORTS BEFORE DRIVING AWAY?

- RAISED ½ WAY WITH THE CRANK HANDLE SECURED IN ITS BRACKET.

- FULLY RAISED WITH THE CRANK HANDLE SECURED IN ITS BRACKET.

- RAISED ¾ WAY WITH CRANK HANDLE REMOVED.

- YOU SHOULD NOT BACK A TRACTOR UNDER A TRAILER UNTIL WHOLE AIR SYSTEM IS:

- EMPTY.

- AT NORMAL PRESSURE.

- BETWEEN 60 AND 80 PSI.

- AIR AND ELECTRICAL LINES FROM THE TRACTOR TO THE TRAILER SHOULD BE:

- PULLED TIGHT, WITH VERY LITTLE SLACK.

- SECURED, BUT WITH ENOUGH SLACK FOR TURNS.

- RESTING ON THE FRAME OF THE TRACTOR.

- AFTER THE TRAILER HAS BEEN COUPLED TO THE TRACTOR, THE TRACTOR PROTECTION CONTROL VALVE SHOULD BE PLACED IN __________ POSITIONX.

- DOWN.

- UP.

- NORMAL.

- WHEN YOU ARE ALLOWED TO GET OUT OF YOUR VEHICLE WHILE BACKING?

- WHEN YOU ARE UNSURE OF YOUR PATH OF TRAVEL.

- WHEN YOUR TRAILER BEGINS TO DRIFTS TO PASSENGER SIDE.

- YOU SHOULD NEVER GET OUT OF YOUR VEHICLE WHEN BACKING.

- WHEN COUPLING, THE PROPER POSITION OF THE FIFTH WHEEL IS:

- LEVEL WITH THE GROUND.

- TILTED DOWN TOWARD THE END OF THE TRAILER.

- TILTED UP TOWARD THE END THE TRAILER.

- HOW SHOULD YOU TEST THE TRACTOR-SEMITRAILER CONNECTION FOR SECURITY?

- LOOK AT IT CAREFULLY.

- ROCK THE TRAILER BACK AND FORTH WITH THE TRAILER BRAKES LOCKED.

- PULL GENTLY FORWARD IN LOW GEAR AGAINST THE LOCKED TRAILER BRAKES THEN LOOK AT IT CAREFULLY.

- YOU SUPPLY AIR TO THE TRAILER TANKS BY:

- PUSHING IN THE TRAILER AIR SUPPLY VALVE.

- PULLING OUT THE TRAILER AIR SUPPLY VALVE.

- CONNECTING THE SERVIVCE LINE GLAD HANDS.

- A TRACTOR WITH A (N) _________ TRAILER REQUIRES THE SHORTEST AMOUNT OF STOPPING DISTANCE.

- EMPTY.

- LIGHTLY LOADED.

- FULLY LOADED.

- WHEN DO YOU NEED TO USE CHOCKS TO PARK A TRAILER?

- NEVER.

- ALWAYS.

- ONLY WHEN IT WILL BE PARKED ON AN INCLINE.

- SEMITRAILERS MADE BEFORE 1975 THAT ARE EQUIPPED WITH THE AIR BRAKES:

- USUALLY NEED A GLAD HAND CONVERTER.

- OFTEN DO NOT HAVE SPRING BRAKES.

- HAVE ONLY A SERVICE AIR LINE.

- THE TRACTOR PROTECTION VALVE WILL CLOSE AND THE TRAILER EMERGENCY BRAKES WILL COME ON WHEN THERE IS A MAJOR LEAK IN THE _______ BRAKE LINE.

- SERVICE.

- PARKING.

- EMERGENCY.

- YOU ARE COUPLING TRACTOR TO A SEMITRAILER. YOU HAVE CONNECTED THE AIRLINES. BEFORE BACKING UNDER THE TRAILER YOU SHOULD:

- SUPPLY AIR TO THE TRAILER SYSTEM, THEM PULL OUT THE AIR SUPPLY KNOB.

- PULL AHEAD TO TEST THE GLAD HANDS CONNECTIONS.

- MAKE SURE THAT THE TRAILER BRAKES ARE OFF.

- THE SAFETY CATCH FOR THE FIFTH WHEEL LOCKING LEVER MUST BE________ FOR A COUPLING TO BE COMPLETE.

- OVER THE LOCKING LEVER.

- STRAIGHT UP.

- THROUGH THE LOCKING LEVER.

- THE ARE TWO THINGS THAT A DRIVER CAN DO TO PREVENT A ROLLOVER. THEY ARE: (1) GO SLOW AROUND TURNS; AND (2):

- KEEP BOTH HANDS FIRMLY ON THE STEERING WHEEL.

- KEEP THE CARGO AS CLOSE TO THE GROUND AS POSSIBLE.

- MAKE SURE THAT THE BRAKES ARE PROPERLY BALANCED.

- LOSS OF AIR PRESSURE IN THE EMERGENCY LINE CAUSES:

- THE RELAY VALVES TO CLOSE.

- AN INCREASE IN AIR PRESSURE TO THE SERVICE BRAKES.

- THE TRAILER’S EMERGENCY BRAKES TO COME ON.

- YOU ARE ABOUT TO BACK YOUR TRACTOR UNDER A SEMITRAILER. THE TRAILER IS AT THE RIGHT HEIGHT WHEN THE:

- THE KINGPIN IS ABOUT 1 ¼  INCHES ABOVE THE FIFTH WHEEL.

- THE END OF THE KING IS EVEN WITH THE TOP OF THE FIFTH WHEEL.

- IT WILL BE RAISED SLIGHTLY WHEN THE TRAILER IS BACKED UNDER IT.

- A DRIVER CROSSES THE AIR LINES WHEN HOOKING UP TO AN OLD TRAILER. WHAT WILL HAPPEN?

- THE HAND VALVE WILL APPLY THE TRACTOR BRAKES INSTEAD OF THE TRAILER BRAKES.

- IF THE TRAILERS HAS NO SPRING BRAKES, HE OR SHE COULD DRIVE AWAY BUT YOU WOULDN’T HAVE TRAILER BRAKES.

- THE BRAKE PEDAL WILL WORK THE TRAILER SPRING BRAKES INSTEAD OF THE AIR BRAKES.

- HOW MUCH SPACE SHOULD BE ALLOWED BETWEEN UPPER AND LOWER FIFTH WHEEL AFTER COUPLING?

- ABOUT ¼ INCH.

- JUST ENOUGH TO SEE LIGHT THROUGH IT.

- NONE.

- AFTER YOU LOCK THE KINGPIN INTO THE FIFTH WHEEL, YOU SHOULD CHECK THE CONNECTION BY:

- PULLING FORWARD 50 FEET, TURNING RIGHT AND THEN LEFT.

- PULLING THE TRACTOR AHEAD SHARPLY TO RELEASE THE TRAILER BRAKES.

- PULLING THE TRACTOR AHEAD GENTLY WITH THE TRAILER BRAKES LOCKED.

- WHILE DRIVING BEHIND OTHER VEHICLE UNDER GOOD CONDITION YOU SHOULD ALLOW AT LEAST ONE SECOND BETWEEN YOUR VEHICLE AND THE VEHICLE AHEAD FOR EACH_____ (HOW MANY) FEET OF YOUR VEHICLE LENGTH?

- 10.

- 20.

- 30.

- OFF-TRACKING OR ‘’CHEATING’’ CAUSES WHICH OF THESE TO FOLLOW THE WIDEST PATH WHILE MAKING A TURN?

- 5-AXLE TRACTOR TOWING A 45 FEET TRAILER.

- 5-AXLE TRACTOR TOWING 27 FEET TRAILERS.

- A BOBTAIL.

- THE AIR LEAKAGE RATE FOR A COMBINATION VEHICLE (ENGINE OFF, BRAKES OFF) SHOULD BE LESS THAN _____ PSI PER MINUTE.

- ½ .

- 2.

- 3.

- WHEN CHECKING THE TRAILER’S EMERGENCY BRAKES, THE TRACTOR PROTECTION CONTROL VALVE SHOULD BE PLACED IN THE _______ POSITION.

- NORMAL.

- EMERGENCY.

- NEUTRAL.

- WHICH OF THESE STATEMENTS IS TRUE?

- THE BRAKES AND SUSPENSION SYSTEM OF COMBINATION VEHICLES WORK BEST WITH A LIGHT LOAD.

- LIGHT VEHICLES NEED MORE BRAKING POWER TO STOP THAN HEAVY ONES.

- ‘’BOBTAIL’’ TRACTORS CAN TAKE LONG TO STOP THAN A COMBINATION VEHICLE LOADED TO MAXIMUM GROSS WEIGHT.

- THERE ARE TWO THINGS THAT A DRIVER CAN DO TO PREVENT A ROLLOVER THEY ARE: (1) KEEP THE CARGO AS CLOSE TO THE GROUND AS POSSIBLE; AND(2):

- MAKE SURE THAT THE BRAKES ARE PROPERLY ADJUSTED.

- KEEP BOTH HANDS FIRMLY ON THE STEERING WHEEL.

- GO SLOW AROUND TURNS.

- WHAT GEAR SHOULD THE TRACTOR TRANSMISSION BE IN AFTER YOU HAVE UNCOUPLED THE TRAILER AND ARE INSPECTING THE TRAILER SUPPORTS?

- NEUTRAL.

- LOW REVERSE.

- HIGH REVERSE.

- HAVING ANTILOCK BRAKES ON ONLY YOUR TRAILER.

- WILL GIVE YOU MORE CONTROL OVER THE VEHICLE WHEN BRAKING.

- WILL INCREASE THE LIKELIHOOD OF A REAR WHEEL POWER UNTIL SKID WHEN BRAKING.

- WILL INCREASE THE LIKELIHOOD OF JACKKNIFING WHEN BRAKING.

- WHEN SHOULD YOU GET OUT OF YOUR VEHICLE WHEN YOUR ARE BACKING WITH A TRAILER?

- WHEN YOU ARE UNSURE OF YOUR PATH OF TRAVEL.

- YOU SHOULD NEVER GET OUT OF YOUR VEHICLE WHEN BACKING.

- WHEN YOUR TRAILER BEGINGS TO DRIFT TO PASSENGER SIDE.

- WHEN UNCOUPLING THE TRAILER, AFTER YOU HAVE SHUT OFF THE TRAILER AIR SUPPLY AND LOCKED THE TRAILER BRAKES, YOU SHOULD:

- BACK UP GENTLY TO ESASE PRESSURE ON THE FIFTH WHEEL LOCKING JAWS.

- IMMEDIATELY PUT ON YOUR TRACTOR PARKING BRAKES.

- BEGIN TO LOWER THE TRAILER LANDING GEAR.

- WHAT WILL HAPPEN IF THE AIR LINES ARE CROSSED WHEN YOU HOOK UP TO AN OLD TRAILER?

- THE HAND VALVE WILL APPLY THE TRACTOR BRAKES INSTEAD OF THE TRAILER BRAKES.

- IF THE TRAILER HAS NO SPRING BRAKES, YOU COULD DRIVE AWAY BUT YOU WOULD NOT HAVE TRAILER BRAKES.

- THE BRAKE LIGHTS WILL NOT COME ON WHEN YOU PRESS THE BRAKE PEDAL.

- THE AIR LEAKAGE RATE FOR A COMBINATION VEHICLE (ENGINE OFF, BRAKES ON) SHOULD BE LESS THAN _______ PSI PER MINUTE.

- 2.

- 3.

- 4.

- THE EARLIEST AND BEST WAY TO RECOGNIZE THAT THE TRAILER HAS STARTED TO SKID IS BY:

- SEEING IT IN YOUR MIRRORS.

- FEEL FOR ‘’PULLING’’ IN THE CAB.

- HEAR THE WHEELS SKIDDING.

- THE AIR LEAKAGE RATE FOR A COMBINATION VEHICLE (ENGINE OFF, BRAKES ON) SHOULD BE LESS THAN ______ PSI PER MINUTE.

- TWO

- THREE.

- FOUR.

- THE FRONT TRAILER SUPPORTS ARE UP AND THE TRAILER IS RESTING ON THE TRACTOR. MAKE SURE:

- THERE IS ENOUGH CLEARANCE BETWEEN THE TOP OF THE TRACTOR TIRES AND NOSE OF THE TRAILER.

- THERE IS ENOUGH CLEARANCE BETWEEN THE TRACTOR FRAME AND THE LANDING GEAR.

- BOTH A AND B ARE CORRECT.

- WHEN DRIVING A SET OF DOUBLES, IT IS NECESSARY TO CLOSE THE ____ SHUT-OFF VALVE IN THE LAST TRAILER.

- FRONT.

- MID-SECTION.

- BACK

- A TRAILER IS MOST LIKELY TO JACKKNIFE WHEN IT IS______.

- LOADED TO FULL CAPACITY.

- OVER LOADED.

- EMPTY.

- WHEN CONNECTING THE GLAD HANDS, PRESS THE TWO SEALS OF THE COUPLERS TOGETHER AT A ________ DEGREE ANGLE TO ONE ANOTHER.

- 45.

- 90.

- 180.

- WHICH PART OF THE KINGPIN IS SHOULD BE LOCKING JAWS CLOSE AROUND?

- THE SHANK.

- THE HEAD.

- THE BASE.

- THE HAND VALVE SHOULD BE USED:

- ONLY WITH A FOOT BRAKE.

- TO TEST THE TRAILER BRAKES.

- AS A PARKING BRAKE.

- OFF-TRACKING OR ‘’CHEATING’’ CAUSES OF WHICH OF THESE TO FOLLOW THE WIDER PATH WHILE MAKING A TURN?

- TRIPPLE TOWING A 45 FEET TRAILER.

- TRACTOR WITH TWO 27 FEET TRAILERS.

- 53-FEET BOBTAIL.

- IF YOUR ANTILOCK BRAKING SYSTEM IS NOT WORKING ON YOUR TRAILER, YOU SHOULD:

- DRIVE AT A LOWER SPEED AND GET THE SYSTEM SERVICED SOON.

- NOT DRIVE UNTIL IT IS REPAIRED.

- DRIVE NORMALLY, BUT GET THE SYSTEM SERVICED SOON.

- YOU ARE COUPLING A TRACTOR TO A SEMITRAILER AND HAVE BACKED UP BUT ARE NOT UNDER IT. WHAT SHOULD YOU HOOK UP BEFORE BAKCING UNDER?

- THE EMERGENCY AND SERVICE AIR LINES.

- THE ELECTRICAL SERVICE CABLE.

- NOTHING. BACK UP AND LOCK THE FIFTH WHEEL.

- YOU WILL KNOW IF THE TRAILER ABS BRAKES ARE NOT WORKING PROPERLY BY:

- YELLOW ABS MALFUNCTION LAMPS ON THE LEFT SIDE OF THE TRAILER STAYS ON.

- YELLOW ABS MALFUNCTION LAMPS ON THE RIGHT SIDE OF THE TRIALER STAYS ON.

- RED MALFUNCTION LAMPS ON THE TRAILER BRAKE CYLINDERS STAYS ON.

- AIR BRAKE EQUIPPED TRAILERS MADE BEFORE 1975.

- OFTEN DO NOT HAVE SPRING BRAKES.

- USUALLY NEED A GLAD HAND CONVERTER.

- CANNOT BE LEGALLY OPERATED ON INTERSTATE HIGHWAYS.

- WHERE SHOUL THE TRACTOR BE WHEN YOU INSPECT LANDING GEAR AFTER UNCOUPLING THE TRAILER?

- WITH THE TRACTOR FRAME UNDER THE TRAILER.

- COMPLETELY CLEARED FROM THE TRAILER.

- WITH THE FIFTH WHEEL DIRECTLY BENEATH THE KINGPIN.

- COMPARED TO A STRAIGHT TRUCK OR BUS, THERE ARE_____ THINGS TO INSPECT IN COMBINATION VEHICLE.

- FEWER.

- THE SAME NUMBER OF.

- MORE.

- YOUR EMERGENCY AIR LINE BREAKS OR GETS PULLED APART WHILE YOU ARE DRIVING. THE LOSS OF PRESSURE WILL CAUSE THE:

- AIR COMPRESSOR TO UNLOAD INSTEAD OF PUMPING AIR.

- EMERGENCY TRAILER BRAKES TO COME ON.

- TRAILER SUPPLY VALVE TO OPEN.

- A TRACTOR-TRAILER VEHICLE COMBINATION IS MOST LIKELY TO ROLL OVER IN TURN WHEN THE CONFIGURATION INCLUDES:

- A 45 FT. AND A 27 FT. TRAILER.

- TRIPLE 27 FT. TRAILERS.

- DOUBLE 45 FT TRAILERS.

- YOU WILL KNOW IF THE TRAILER YOU ARE DRIVING HAS ANTILOCK BRAKES BY:

- YELLOW ABS MALFUNCTION LAMPS ON THE LEFT SIDE OF THE TRAILER.

- RED MALFUNCTION LAMPS ON THE TRAILER BRAKE CYLINDERS.

- YELLOW ABS MALFUNCTION LAMPS ON THE RIGHT SIDE OF THE TRAILER.

- IF YOU CANNOT MAKE A TURN WITHOUT ENTERING ANOTHER TRAFFIC LANE:

- YOU SHOULD TURN WIDE WHEN YOU START THE TURN.

- YOU SHOULD TURN WIDE AS YOU COMPLETE THE TURN.

- NOT TO MAKE A TURN. GO TO ANOTHER PLACE WHERE YOU WONT HAVE TO CROSS INTO ANOTHER LANE.

- AFTER YOU SUPPLY AIR TO THE TRAILER, MAKE SURE THE AIR LINES ARE NOT CROSSED AND THE TRAILER BRAKE ARE WORKING. THIS IS DONE BY:

- TURNING ON THE PARKING BRAKES FROM THE CAB.

- WATCHING YOUR MIRRORS TO SEE IF THE TRAILERS LIGHTS COME ON.

- APPLYING AND RELEASING THE TRAILER BRAKES AND LISTENING FOR BRAKE SOUNDS.

- TO STOP A TRAILER SKID YOU SHOULD:

- USE THE TRAILER HAND BRAKES.

- RELEASE THE SERVICE BRAKES.

- COUNTERSTEER.

- WHEN COUPLING, THE PROPER POSITION OF THE FIFTH WHEEL IS:

- LEVEL WITH THE GROUND.

- TILTED UP TOWARD THE END THE TRACTOR.

- TILTED TOWARD THE END THE TRACTOR.

- TO UNLOCK THE FIFTH WHEEL, PULL THE RELEASE HANDLE TO THE _____ POSITION.

- NEUTRAL.

- CLOSED.

- OPEN.

- WHY SHOULD YOU LOCK THE TRACTOR GLAD HANDS TO EACH OTHER (OR DUMMY COUPLERS) WHEN YOU ARE NOT TOWING A TRAILER?

- THE CONNECTED BRAKE CIRCUIT BECOMES A BACK UP AIR TANK.

- IT WILL KEEP DIRT AND WATER OUT OF THE LINES.

- IF YOU DIDN’T, YOU COULD NEVER BUILD SYSTEM PRESSURE.

- AIR LINES ON A COMBINATION VEHICLE ARE OFTEN COLORED TO PREVENT THEN FROM MIXING UP. THE EMERGENCY LINE IS __________; THE SERVICE LINE IS ________.

- RED, BLUE.

- BLACK, YELLOW.

- BLUE, RED.

- IF YOU CANNOT MAKE A TURN WITHOUT ENTERING ANOTHER TRAFFIC LINE, YOU SHOULD:

- TURN WIDE AS YOU COMPLETE YOUR TURN.

- NOT MAKE THAT TURN- GO TO ANOTHER PLACE WHERE YOU WONT HAVE TO CROSS INTO ANOTHER LINE.

- TURN WIDE BEFORE YOU START YOUR TURN.

- YOU ARE DRIVING A COMBINATION VEHICLE WHEN THE TRAILER BREAKS AWAY, PULLING APART BOTH AIR LINES. YOU WOULD EXPECT THE TRAILER BRAKES TO COME ON AND:

- THE TRAILER SUPPLY VALVE TO STAY OPEN.

- THE TRACTOR TO LOSE ALL AIR PRESSURE.

- THE TRACTOR PROTECTION VALVE TO CLOSE.

- WHEN SHOULD YOU USE THE HAND VALVE TO PARK A COMBINATION VEHICLE?

- TO PARK AT LOADING DOCKS.

- TO PARK ON A GRADE.

- NEVER.

- TO UNLOCK THE FIFTH WHEEL LOCKING LEVER SHOULD BE PLACED IN THE _____ POSITION.

- NEUTRAL.

- CLOSED.

- OPEN.

- GLAD HANDS ARE USED FOR CONNECTING THE:

- ELECTRIC LINES FROM THE TRACT TO THE TRAILER.

- KINGPIN FROM THE TRAILER TO THE LOCKING JAWS OF THE FIFTH WHEEL.

- SERVICE AND EMERGENCY AIR LINES FROM THE VEHICLE TO THE TRAILER.

- AFTER YOU HAVE COUPLED THE TRAILER YOU SHOULD HAVE TO RAISE THE LANDING GEAR BY USING:

- LOW GEAR.

- INTERMEDIATE.

- HIGH GEAR.

- THE SAFEST WAY TO MAKE TURN WITHOUT ENTERING ANOTHER TRAFFIC LANE IS:

- YOU SHOULD TURN WIDE BEFORE YOU START THE TURN.

- YOU SHOULD TURN WIDE AS YOU COMPLETE THE TURN.

- YOU SHOULD NOT MAKE A TURN AND MOVE TO ANOTHER PLACE WHERE YOU CAN MAKE A TURN WITHOUTH CROSSING OTHER LANE.